[Vibrating Motor]

↓

[MPU6050 Sensor] ← measures acceleration in 3 axes

↓

[Arduino/ESP32] ← reads sensor data at 200+ Hz

↓

[AI Model] ← classifies vibration pattern

↓

[Alert System] ← displays health status

Step-by-Step Workflow

Phase 1: Data Collection (Days 1-2)

- Hardware Setup

  - Connect MPU6050 accelerometer to Arduino

  - Secure a small DC motor to a surface

  - Position sensor on/near the motor housing

- Create Three Fault Conditions

  - Normal: Motor running smoothly

  - Imbalance: Tape a coin/weight to the motor shaft (simulates worn bearing)

  - Loose Mount: Loosen screws (simulates loose foundation)

- Collect Data

  - Record 2-3 minutes per condition

  - Capture X, Y, Z acceleration values at 200 Hz

  - Save as CSV: timestamp, accel_x, accel_y, accel_z, label

Why this matters: Real-world data with real physics. No simulation can replicate mechanical chaos perfectly.

Phase 2: Feature Engineering (Days 3-4)

Raw vibration data is noisy. You need to extract meaningful patterns:

- Frequency Analysis (FFT)

  - Convert time-domain signals → frequency-domain

  - Healthy motors show peaks at specific frequencies (e.g., 60 Hz rotation speed)

  - Faults create "harmonic" peaks at 2x, 3x, 4x the rotation speed

- Statistical Features

  - RMS (root mean square) — overall energy

  - Peak-to-peak amplitude — severity of vibration

  - Kurtosis — detects sudden shocks

- Upload to Edge Impulse

  - Edge Impulse automates FFT and feature extraction

  - Visual interface for labeling data

  - Trains models optimized for microcontrollers

Phase 3: Model Training (Days 3-4)

Two approaches:

Option A: Edge Impulse (Recommended for speed)

- Upload your CSV files

- Use their spectral analysis block (automatic FFT)

- Train a 1D-CNN or Random Forest

- Export as Arduino library

Option B: Python (More control)

python

# Pseudocode

import numpy as np

from sklearn.ensemble import RandomForestClassifier

# Extract features

features = compute_fft(vibration_data)

labels = ['normal', 'imbalance', 'loose']

# Train model

model = RandomForestClassifier()

model.fit(features, labels)

```

**Expected Accuracy:** 85-95% (even simple models work well because vibration signatures are distinct)

---

### **Phase 4: Real-Time Deployment (Days 5-6)**

**Hardware Path:**

1. Export Edge Impulse model as Arduino library

2. Flash to ESP32

3. Run inference at 10-20 Hz (predictions per second)

4. Send results via serial/WiFi to dashboard

**Software Path:**

1. Arduino streams sensor data via USB

2. Python script reads serial port

3. Runs model inference

4. Displays results in Streamlit dashboard

**Example Dashboard:**

```

🟢 MACHINE HEALTH: NORMAL

━━━━━━━━━━━━━━━━━━━━━━━━━

Vibration Level: 0.2g

Last Fault: None

Uptime: 47 hours

Phase 5: Testing & Demo

The "Blind Test" (Proves AI works):

- Hide the motor under a box

- Secretly introduce a fault (loosen a screw)

- Show the AI detecting it without human observation

- This demonstrates the system responds to sensor data, not visual cues

Presentation Tips:

- Live demo with real hardware

- Show the dashboard catching a fault in real-time

- Compare AI predictions vs. visual inspection

- Discuss cost savings (avoiding downtime)

Required Materials

Hardware Arduino Uno/ESP32

- MPU6050 accelerometer

- Small DC motor

- Jumper wires, breadboard

Software (Free):

- Arduino IDE

- Edge Impulse (free tier)

- Python + Streamlit

Expected Outcomes

By Day 8, you'll have:

- A working prototype that detects 3 fault types

- A live dashboard showing machine health

- Data proving AI accuracy (confusion matrix)

- A unique project that demonstrates AI + hardware integration

This showcases skills in:

- Embedded systems

- Signal processing

- Machine learning deployment

- Real-world problem-solving